Quick Start Guide for starting Soft-AP mode

(A) How to start Soft-AP mode:

(1) disable network management or other wireless tools, e.g. wpa_supplicant

(2) uncompress the driver and then compile the driver

./make

(3) insmod 8192cu.ko

(4) ifconfig wlan0 192.168.0.1 (using the static ip for testing)

(5) compile HOSTAP, unpack "wpa_supplicant_hostapd-0.8_rtw_20110524.zip"

in the folder (wpa_supplicant_hostapd-0.8\hostapd)

./make

(6) start hostapd daemon:

./hostapd rtl_hostapd.conf -B

(B) Configure file for Soft-AP mode setting:

(1) rtl_hostapd.conf is the configure file for functions setting.

(2) the major variable setting in the rtl_hostapd.conf configure file,

(i) basic configuration

interface=wlan0

ssid=rtwap

channel=6

(ii) security mode configuration

# This field is a bit field that can be used to enable WPA (IEEE 802.11i/D3.0)

# and/or WPA2 (full IEEE 802.11i/RSN):

# bit1 = IEEE 802.11i/RSN (WPA2) (dot11RSNAEnabled)

wpa=2

#wpa_psk=0123456789abcdef0123456789abcdef0123456789abcdef0123456789abcdef

wpa_passphrase=87654321

wpa_key_mgmt=WPA-PSK

wpa_pairwise=CCMP

(iii) IEEE 802.11n related configuration

# ieee80211n: Whether IEEE 802.11n (HT) is enabled

# 0 = disabled (default)

# 1 = enabled

ieee80211n=1

(C) check the station connected to softap using hostapd_cli:

./hostapd_cli all_sta

(D) How to start WPS process as internal registrar?

1. for PIN code = 12345670

$>./hostapd_cli wps_pin any 12345670

2. for PBC

$>./hostapd_cli wps_pbc